# Buffer Preparation SOP

| Document Number: | SOP-BP-2026-042 |
| --- | --- |
| Effective Date: | January 15, 2026 |
| Prepared By: | Dr. Sarah Chen |
| Department: | Upstream Process Development |

## 1. Purpose

This SOP describes the preparation of Tris-HCl buffer (50 mM, pH 7.4) for use in downstream purification steps of monoclonal antibody production at Acme Therapeutics.

## 2. Scope

This procedure applies to all Process Development scientists and associates at Acme Therapeutics who perform buffer preparation for the AAV Production Campaign Q1 project.

## 3. Materials and Equipment

| Item | Specification | Lot Number |
| --- | --- | --- |
| Tris Base | ACS Grade, Fisher Scientific | LOT-2026-A1234 |
| Hydrochloric Acid (1N) | Certified ACS, Fisher | LOT-2026-B5678 |
| Purified Water | Milli-Q, >18 MΩ·cm | N/A |
| pH Meter | Mettler Toledo SevenEasy | EQ-PH-007 |

## 4. Procedure

| Step | Description | Duration (min) | Operator | Status |
| --- | --- | --- | --- | --- |
| 1 | Weigh 6.057 g Tris Base on analytical balance | 5 | SC | Complete |
| 2 | Add Tris Base to 800 mL purified water in 1L beaker | 3 | SC | Complete |
| 3 | Stir with magnetic stir bar until fully dissolved | 15 | SC | Complete |
| 4 | Adjust pH to 7.4 using 1N HCl (approx 30 mL added) | 10 | SC | Complete |
| 5 | Bring total volume to 1000 mL with purified water | 2 | SC | Complete |

## 5. Results

| Final pH: | 7.41 |
| --- | --- |
| Final Volume: | 1000 mL |
| Visual Appearance: | Clear, colorless solution |
| Conductivity: | 5.2 mS/cm |

## 6. Approval

| Role | Name | Date |
| --- | --- | --- |
| Prepared By | Dr. Sarah Chen | 2026-01-15 |
| Reviewed By | Dr. James Park | 2026-01-16 |