# Batch Record

## CHO Cell Culture — mAb Production

| Batch #: | BR-2026-0087 | Product: | mAb-X Therapeutic |
| --- | --- | --- | --- |
| Campaign: | Q1 Commercial Mfg | Scale: | 2000L Single-Use |
| Start Date: | 2026-03-10 06:00 | Target End: | 2026-03-15 18:00 |

## Procedure

| Step | Description | Expected Result | Actual Result | Operator | Date/Time |
| --- | --- | --- | --- | --- | --- |
| 1 | Prepare CD CHO medium: add 8 mM L-glutamine and 6 g/L glucose to base medium | Clear solution, pH 7.0 +/- 0.2 | Clear, pH 7.05 | J. Williams | 03/10 06:15 |
| 2 | Set up bioreactor: pH 7.0, DO 40%, temp 37C, agitation 150 RPM. Verify probes. | All setpoints confirmed | All confirmed | J. Williams | 03/10 07:00 |
| 3 | Inoculate with CHO-mAb cells. Target: 0.3e6 cells/mL in 2000L. | VCD 0.25-0.35e6/mL | VCD 0.31e6/mL | K. Lee | 03/10 08:30 |
| 4 | Day 3 feed: add Feed A at 3% v/v. Record volume. | Feed volume: 60L | 60.2L added | J. Williams | 03/13 08:00 |
| 5 | Day 3 temp shift: reduce 37C to 33C. Monitor 30 min. | 33C +/- 0.5C in 30 min | 33.1C at 30 min | J. Williams | 03/13 09:00 |
| 6 | Day 5 harvest: transfer to harvest vessel when viability < 85%. | Viability > 80% at harvest | Viability 82% | K. Lee | 03/15 14:00 |
| 7 | Final sampling: 3x50mL for QC (titer, glycan, HCP). | Samples labeled and submitted | 3 samples to QC | K. Lee | 03/15 15:30 |

## Sign-off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Executed By | J. Williams | JW | 2026-03-15 |
| Verified By | K. Lee | KL | 2026-03-15 |
| Approved By | Dr. R. Patel | RP | 2026-03-16 |