# Batch Record — mAb-X CHO Cell Culture

| Batch Number: | BR-2026-0042 | Product: | mAb-X |
| --- | --- | --- | --- |
| Campaign: | AAV Campaign Q1 | Scale: | 2000L |
| Start Date: | 2026-02-10 08:00 | End Date: | 2026-02-14 16:30 |

## Process Steps

| Step | Operation | Parameters | Duration | Operator | Initials/Date |
| --- | --- | --- | --- | --- | --- |
| 1 | Media Preparation | CD CHO medium + 8 mM L-glutamine | 45 min | Upstream Lead | JW 02/10 |
| 2 | Bioreactor Setup | pH 7.0, DO 40%, Temp 37°C, Agitation 150 RPM | 30 min | Upstream Lead | JW 02/10 |
| 3 | Seeding | Target VCD: 0.3 x 10^6 cells/mL, Volume: 1500L | 20 min | Scientist One | KL 02/10 |
| 4 | Growth Phase Monitoring | Daily VCD, viability, glucose, lactate | 96 hrs | Scientist One | KL 02/10-14 |
| 5 | Temperature Shift | Reduce to 33°C at VCD > 6 x 10^6 | 10 min | Upstream Lead | JW 02/12 |
| 6 | Feed Addition | Feed A: 3% v/v on Days 3, 5, 7 | 15 min each | Scientist One | KL 02/12-14 |
| 7 | Harvest | Final VCD: 12.4 x 10^6, Viability: 89% | 120 min | Downstream Lead | MP 02/14 |
| 8 | Centrifugation | 4000 RPM, 30 min, Beckman J6-MI | 45 min | Downstream Lead | MP 02/14 |

## In-Process Controls

| Test | Specification | Result | Pass/Fail |
| --- | --- | --- | --- |
| pH | 6.8 - 7.2 | 7.01 | Pass |
| Dissolved O2 | 35 - 45% | 40.2% | Pass |
| Viability at Harvest | > 80% | 89% | Pass |
| Sterility Check | No growth | No growth | Pass |

## Deviations

Deviation DEV-2026-003: Temperature overshoot to 37.5°C for 12 minutes during temperature shift on 02/12. Root cause: PID controller lag. Corrective action: Recalibrated controller. Impact assessment: No impact on product quality (VCD and viability within spec). Approved by: Dr. James Park, 02/13/2026.

## Sign-off

| Role | Signature | Date |
| --- | --- | --- |
| Manufacturing | J. Williams | 2026-02-14 |
| QA Review | Dr. J. Park | 2026-02-15 |